Packet Scheduling Simulation — NS-3

Wireless Networks Semester Project

Simulation Code Files

NS-3 Version: ns-3-dev  |  Language: C++ & Python

# Overview

This document contains the complete, verified simulation source code for the Packet Scheduling Techniques for QoS Optimization project. All code was tested and run successfully on Ubuntu with NS-3 (ns-3-dev).

The simulation compares three scheduling algorithms — FIFO, Priority Queuing (PQ), and Weighted Fair Queuing (WFQ) — on a bottleneck dumbbell topology with 4 UDP senders, 1 router, and 1 receiver.

# How to Run

## Build the Simulation

cd ~/ns-3-dev

./ns3 build

## Run Each Scheduler

# FIFO

./ns3 run "scratch/packet-scheduling/scheduling --schedType=FIFO" 2>&1 | grep -E "====|FlowID|^[0-9]"

# Priority Queuing

./ns3 run "scratch/packet-scheduling/scheduling --schedType=PQ"  2>&1 | grep -E "====|FlowID|^[0-9]"

# Weighted Fair Queuing

./ns3 run "scratch/packet-scheduling/scheduling --schedType=WFQ" 2>&1 | grep -E "====|FlowID|^[0-9]"

## File Location

~/ns-3-dev/scratch/packet-scheduling/scheduling.cc

# File 1: scheduling.cc  (NS-3 C++ Simulation)

This is the main simulation file. It supports all three schedulers via the --schedType command-line argument. Place it at: scratch/packet-scheduling/scheduling.cc

#include "ns3/core-module.h"

#include "ns3/network-module.h"

#include "ns3/internet-module.h"

#include "ns3/point-to-point-module.h"

#include "ns3/applications-module.h"

#include "ns3/traffic-control-module.h"

#include "ns3/flow-monitor-module.h"

#include <iomanip>

using namespace ns3;

NS_LOG_COMPONENT_DEFINE ("PacketScheduling");

int main (int argc, char *argv[])

{

std::string schedType = "FIFO";

CommandLine cmd;

cmd.AddValue ("schedType", "Scheduling type: FIFO, PQ, WFQ", schedType);

cmd.Parse (argc, argv);

Time::SetResolution (Time::NS);

NodeContainer senders;

senders.Create (4);

NodeContainer router;

router.Create (1);

NodeContainer receiver;

receiver.Create (1);

PointToPointHelper senderLink;

senderLink.SetDeviceAttribute ("DataRate", StringValue ("10Mbps"));

senderLink.SetChannelAttribute ("Delay", StringValue ("2ms"));

PointToPointHelper bottleneck;

bottleneck.SetDeviceAttribute ("DataRate", StringValue ("5Mbps"));

bottleneck.SetChannelAttribute ("Delay", StringValue ("5ms"));

NetDeviceContainer senderDevices[4];

for (int i = 0; i < 4; i++)

senderDevices[i] = senderLink.Install (senders.Get (i), router.Get (0));

NetDeviceContainer routerReceiverDevices =

bottleneck.Install (router.Get (0), receiver.Get (0));

InternetStackHelper stack;

stack.Install (senders);

stack.Install (router);

stack.Install (receiver);

TrafficControlHelper tch;

if (schedType == "PQ")

tch.SetRootQueueDisc ("ns3::PrioQueueDisc");

else if (schedType == "WFQ")

tch.SetRootQueueDisc ("ns3::FqCoDelQueueDisc");

else

tch.SetRootQueueDisc ("ns3::FifoQueueDisc");

tch.Install (routerReceiverDevices.Get (0));

Ipv4AddressHelper address;

for (int i = 0; i < 4; i++)

{

std::ostringstream subnet;

subnet << "10.1." << (i + 1) << ".0";

address.SetBase (subnet.str ().c_str (), "255.255.255.0");

address.Assign (senderDevices[i]);

}

address.SetBase ("10.1.5.0", "255.255.255.0");

Ipv4InterfaceContainer rrIfaces = address.Assign (routerReceiverDevices);

Ipv4GlobalRoutingHelper::PopulateRoutingTables ();

Ipv4Address receiverAddr = rrIfaces.GetAddress (1);

uint16_t basePort = 5000;

// Install UDP Sinks on receiver

for (int i = 0; i < 4; i++)

{

PacketSinkHelper sink ("ns3::UdpSocketFactory",

InetSocketAddress (receiverAddr, basePort + i));

ApplicationContainer sinkApp = sink.Install (receiver.Get (0));

sinkApp.Start (Seconds (0.0));

sinkApp.Stop (Seconds (10.0));

}

// TOS: flows 0,1 = high priority (0x10), flows 2,3 = low (0x00)

uint8_t tosValues[] = {0x10, 0x10, 0x00, 0x00};

// Install UDP Sources on senders

for (int i = 0; i < 4; i++)

{

OnOffHelper source ("ns3::UdpSocketFactory",

InetSocketAddress (receiverAddr, basePort + i));

source.SetConstantRate (DataRate ("2Mbps"));

source.SetAttribute ("PacketSize", UintegerValue (1024));

source.SetAttribute ("OnTime",

StringValue ("ns3::ConstantRandomVariable[Constant=1]"));

source.SetAttribute ("OffTime",

StringValue ("ns3::ConstantRandomVariable[Constant=0]"));

if (schedType == "PQ")

source.SetAttribute ("Tos", UintegerValue (tosValues[i]));

ApplicationContainer srcApp = source.Install (senders.Get (i));

srcApp.Start (Seconds (1.0));

srcApp.Stop (Seconds (9.0));

}

// Enable FlowMonitor

FlowMonitorHelper flowmon;

Ptr<FlowMonitor> monitor = flowmon.InstallAll ();

Simulator::Stop (Seconds (10.0));

Simulator::Run ();

// Collect and print results

monitor->CheckForLostPackets ();

Ptr<Ipv4FlowClassifier> classifier =

DynamicCast<Ipv4FlowClassifier> (flowmon.GetClassifier ());

std::map<FlowId, FlowMonitor::FlowStats> stats = monitor->GetFlowStats ();

NS_LOG_UNCOND ("\n===== " << schedType << " Scheduler Results =====");

NS_LOG_UNCOND (std::left

<< std::setw(8)  << "FlowID"

<< std::setw(16) << "DstPort"

<< std::setw(12) << "Priority"

<< std::setw(15) << "Throughput"

<< std::setw(16) << "AvgDelay(ms)"

<< std::setw(10) << "PDR(%)");

int idx = 0;

for (auto& flow : stats)

{

Ipv4FlowClassifier::FiveTuple t = classifier->FindFlow (flow.first);

double throughput = flow.second.rxBytes * 8.0 / 8.0 / 1e6;

double avgDelay   = 0;

uint32_t rx = flow.second.rxPackets;

uint32_t tx = flow.second.txPackets;

double pdr  = (tx > 0) ? (double)rx / tx * 100.0 : 0;

if (rx > 0)

avgDelay = flow.second.delaySum.GetSeconds () / rx * 1000.0;

std::string prio = "Equal";

if (schedType == "PQ")  prio = (idx < 2) ? "High" : "Low";

if (schedType == "WFQ") prio = "Fair";

NS_LOG_UNCOND (std::left

<< std::setw(8)  << flow.first

<< std::setw(16) << t.destinationPort

<< std::setw(12) << prio

<< std::setw(15) << std::fixed << std::setprecision(3) << throughput

<< std::setw(16) << avgDelay

<< std::setw(10) << pdr);

idx++;

}

Simulator::Destroy ();

return 0;

}

# File 2: plot_results.py (Python Results & Graph Generator)

Run this script after completing all three simulations to generate the comparison bar chart (scheduling_results.png) and CSV data file (scheduling_results.csv). Run from the ~/ns-3-dev directory.

## Run Command

python3 plot_results.py

## Script Code

import matplotlib

matplotlib.use('Agg')

import matplotlib.pyplot as plt

import numpy as np

flows = ['Flow1 (5000)', 'Flow2 (5001)', 'Flow3 (5002)', 'Flow4 (5003)']

data = {

'FIFO': {

'throughput': [1.833, 1.833, 1.206, 0.735],

'delay':      [1489.315, 1491.001, 1299.033, 939.299],

'pdr':        [89.196, 89.196, 58.679, 35.791],

},

'PQ': {

'throughput': [2.055, 2.055, 0.761, 0.736],

'delay':      [176.197, 177.883, 3973.958, 3913.506],

'pdr':        [100.0, 100.0, 37.020, 35.842],

},

'WFQ': {

'throughput': [1.291, 1.290, 1.291, 1.293],

'delay':      [327.800, 326.603, 328.652, 327.323],

'pdr':        [62.826, 62.775, 62.826, 62.929],

},

}

colors = {'FIFO': '#e74c3c', 'PQ': '#3498db', 'WFQ': '#2ecc71'}

x = np.arange(len(flows))

width = 0.25

schedulers = ['FIFO', 'PQ', 'WFQ']

fig, axes = plt.subplots(1, 3, figsize=(16, 6))

fig.suptitle('Packet Scheduling Algorithms: QoS Comparison',

fontsize=15, fontweight='bold')

for ax, metric, ylabel, title in zip(

axes,

['throughput', 'delay', 'pdr'],

['Mbps', 'ms', '%'],

['Throughput (Mbps)', 'Average Delay (ms)', 'Packet Delivery Ratio (%)']):

for i, s in enumerate(schedulers):

ax.bar(x + i*width, data[s][metric], width,

label=s, color=colors[s], alpha=0.85)

ax.set_title(title, fontweight='bold')

ax.set_xticks(x + width)

ax.set_xticklabels(flows, rotation=10, fontsize=8)

ax.set_ylabel(ylabel)

ax.legend()

ax.grid(axis='y', alpha=0.3)

if metric == 'pdr':

ax.set_ylim(0, 110)

plt.tight_layout()

plt.savefig('scheduling_results.png', dpi=150, bbox_inches='tight')

print("Graph saved: scheduling_results.png")

import csv

with open('scheduling_results.csv', 'w', newline='') as f:

w = csv.writer(f)

w.writerow(['Scheduler','Flow','DstPort',

'Throughput_Mbps','AvgDelay_ms','PDR_percent'])

ports = [5000, 5001, 5002, 5003]

for s in schedulers:

for i in range(4):

w.writerow([s, i+1, ports[i],

data[s]['throughput'][i],

data[s]['delay'][i],

data[s]['pdr'][i]])

print("CSV saved: scheduling_results.csv")

# Simulation Parameters Summary

| Parameter | Value |
| --- | --- |
| Number of Senders | 4 |
| Number of Routers | 1 (bottleneck) |
| Number of Receivers | 1 |
| Sender Link Speed | 10 Mbps |
| Sender Link Delay | 2 ms |
| Bottleneck Link Speed | 5 Mbps |
| Bottleneck Link Delay | 5 ms |
| Traffic Protocol | UDP (OnOff Application) |
| Per-flow Send Rate | 2 Mbps (total offered: 8 Mbps) |
| Packet Size | 1024 bytes |
| Simulation Duration | 10 seconds (traffic: t=1s to t=9s) |
| PQ High-Priority TOS | 0x10 (Flows 1 & 2) |
| PQ Low-Priority TOS | 0x00 (Flows 3 & 4) |
| WFQ Implementation | ns3::FqCoDelQueueDisc |
| Metrics Collected | Throughput, Avg Delay, PDR |

# Verified Results Summary

| Scheduler | Flow | Port | Throughput (Mbps) | Avg Delay (ms) | PDR (%) |
| --- | --- | --- | --- | --- | --- |
| FIFO | 1 | 5000 | 1.833 | 1489.3 | 89.2 |
| FIFO | 2 | 5001 | 1.833 | 1491.0 | 89.2 |
| FIFO | 3 | 5002 | 1.206 | 1299.0 | 58.7 |
| FIFO | 4 | 5003 | 0.735 | 939.3 | 35.8 |
| PQ (High) | 1 | 5000 | 2.055 | 176.2 | 100.0 |
| PQ (High) | 2 | 5001 | 2.055 | 177.9 | 100.0 |
| PQ (Low) | 3 | 5002 | 0.761 | 3974.0 | 37.0 |
| PQ (Low) | 4 | 5003 | 0.736 | 3913.5 | 35.8 |
| WFQ | 1 | 5000 | 1.291 | 327.8 | 62.8 |
| WFQ | 2 | 5001 | 1.290 | 326.6 | 62.8 |
| WFQ | 3 | 5002 | 1.291 | 328.7 | 62.8 |
| WFQ | 4 | 5003 | 1.293 | 327.3 | 62.9 |